台科大

PA1

Computer Organization

周 柏宇 2024/3/28

目錄

Multiplier	3

CompMultiplier	3

Description	3

Details	4

ALU	6

Description	6

Detail	6

Multiplicand	7

Description	7

Detail	7

Control	8

Description	9

Detail	9

Product	10

Description	10

Detail	11

Divider	12

CompDivider	12

Description	13

Detail	13

ALU	14

Description	14

Detail	14

Divisor	14

Description	15

Detail	15

Control	16

Description	17

Detail	17

Remainder	18

Description	18

Detail	19

Test	20

Multiplier	20

tb_CompMultiplier	20

tb_ALU	21

tb_Control	22

tb_Multiplicand	23

tb_Product	23

Divider	25

tb_Control	25

tb_ALU	26

tb_Divisor	27

tb_Remainder	28

tb_CompDivider	29

Conclusion and Insights	31

Multiplier

CompMultiplier

Description

This module is to connect all the small components to form a functional multiplier.

Details

Line 1~9

I/O Interface.

| Signal Symbol | Signal Name | Signal Description |
| --- | --- | --- |
| Prod | 64-bit calculation result | This signal is set as an output before the Rdy signal is activated to forbid the testbench to read the previous calculation result. |
| Rdy | completion signal | The “high” level represents the system has completed multiplication and maintained the result. |
| Mult | 32-bit multiplicand | The signal is generated by the testbench and updated when the Rst signal is “high”. |
| Mul | 32-bit multiplier | The signal is generated by the testbench and updated when the Rst signal is “high”. |
| Run | execution signal | The system performs multiplication as the Run signal is “high” level. |
| Rst | initialization signal | The “high” level indicates that the system is initialized before multiplication, and the output results are set to zero. This signal has the highest priority and is generated by the testbench. |
| clk | clock signal | Periodic square waves are generated by the testbench for synchronizing each signal with the drive system. |

Line 11~14

Wires used in this module.

| Wire Symbol | Wire Name | Wire Description |
| --- | --- | --- |
| Hi | HI register | Upper 32 bits of the product. Used to pass data to ALU. |
| ALU_result | 33 bits ALU result. This sees overflow as the 33th bit. | The computation result of ALU. |
| Mult_out | 32-bit multiplicand | The signal is generated by the testbench and updated when the Rst signal is “high”. |
| counting | Counting flag | Tell Control to start. This will avoid racing condition. |

Line 16~43

Component instances.

| Instance Name | Component Name | Instance Description |
| --- | --- | --- |
| ALU_inst | ALU instance | src_1 gets data from Mult_out, src_2 gets data from Hi, result is the output of this ALU, connected to ALU_result. |
| Multiplicand_inst | Multiplicand instance | Mult_in gets data from Mult, Mult_out sets output to Mult_out. |
| Control_inst | Control instance | counting gets data from Product, rst gets data from Rst, clk gets data from clk, ready sets output to Rdy. |
| Product_inst | Product instance | clk gets data from clk, rst gets data from Rst, run gets data from Run, Mul gets data from Mul on the posedge of Run, ALU_result gets data from ALU_result, Hi outputs data to Hi, Prod generates output to Prod. And last but not least it gives counting to Control. |

ALU

Description

This module provides basic arithmetic functions that fulfills multiplier’s needs.

Detail

Line 1~5

I/O Interface.

| Signal Symbol | Signal Name | Signal Description |
| --- | --- | --- |
| src_1 | 32-bits Source 1 | addend |
| src_2 | 32-bits Source 2 | augend |
| result | 33-bits addition result | Sum with 1 overflow bit |

Line 6

Wire connection that represents result = src_1 + src_2.

Multiplicand

Description

Dummy module. Mult signal is given all along between the 2 posedge of Rst. There is no need to do any other operation.

Detail

Line 1~4

I/O Interface.

| Signal Symbol | Signal Name | Signal Description |
| --- | --- | --- |
| Mult_in | In direction of Mult | In |
| Mult_out | Out direction of Mult | out |

Line 5

Wire connection that denotes Mult_out = Mult_in.

Control

Description

A counter with a flag.

Detail

Line 1~6

I/O Interface.

| Signal Symbol | Signal Name | Signal Description |
| --- | --- | --- |
| counting | Counting flag | The flag telling Control to count. |
| rst | initialization signal | The “high” level indicates that the system is initialized before multiplication, and the output results are set to zero. This signal has the highest priority and is generated by the testbench. |
| clk |  | Periodic square waves are generated by the testbench for synchronizing each signal with the drive system. |
| ready | Ready flag | Signify the end of the sequence of operation. |

Line 11~18

Use reset register to hold rst signal between 2 posedge clk to avoid racing.

Line 20~31

When running count 32 negedges of clk then set ready flag to notify other component to stop or fetch data.

Product

Description

Main module. I use 65 bits representing product in order to handle overflowing result from ALU. The computation is executed on posedge clk to avoid racing condition. And I use counting, which is a flag to tell Control to count. This flag will effective avoid racing.

Detail

Line 1~10

I/O Interface.

| Signal Symbol | Signal Name | Signal Description |
| --- | --- | --- |
| clk | clock | Clock |
| rst | reset | Reset the init state |
| run | Execution signal | Run operations when high |
| Mul | 32-bits multiplicand | Init will put this to LO |
| ALU_result | 33-bits ALU result | With 1 more carry bit |
| Hi | HI | Upper 32 bits |
| Prod | production | 64-bits result |
| counting | Counting flag | Used to avoid racing |

Line 11~12

Registers

| Register Symbol | Register Name | Register Description |
| --- | --- | --- |
| state | state | FSM state |
| product | 65-bits product | 1 more bit for overflow result generated by ALU. |

Line 14~16

Wire connection of Hi, Prod and counting.

Line 19~22

Reset.

Line 24~38

FSM

| State Symbol | State Name | State Description |
| --- | --- | --- |
| 0 | Init | HI = 0, LO = Mul |
| 1 | run | Examine LSB to do predefined operations. |

Line 30~36

If LSB is set, write ALU_result to HI and shift right 1 bit.

Else, shift right 1 bit.

Divider

CompDivider

Description

This module is to connect all the small components to form a functional divider.

Detail

Line 1~10

I/O Interface.

| Signal Symbol | Signal Name | Signal Description |
| --- | --- | --- |
| Q | 32-bit Quotient | Quotient result |
| R | 32-bit Reminder | Remainder result |
| Rdy | completion signal | Iff it’s set the, result is valid |
| Dvnd | 32-bit Dividend | Input |
| Dvsr | 32-bit Divisor | Input |
| Run | execution signal | Start execute when high |
| Rst | initialization signal | Reset |
| clk | clock signal | clock |

Line 11~14

Wires used in this module.

| Wire Symbol | Wire Name | Wire Description |
| --- | --- | --- |
| ALU_result | 33-bits ALU out | With 1 carry bit |
| Dvsr_out | Divisor out | Dummy connection |
| Hi | HI register | Upper 32 bits |
| counting | Counting flag | Used to avoid racing. |

Line 16~44

Component instances.

| Instance Symbol | Instance Name | Instance Description |
| --- | --- | --- |
| ALU_inst | ALU instance | Output to ALU_result considering R, Dvsr_out |
| Divisor_inst | Divisor instance | Dummy |
| Control_inst | Control instance | Counter with Rdy flag. |
| Remainder_inst | Remainder instance | 1 clk, 2 flags, 2 in, 4 out. For more detail click me. |

ALU

Description

This module provides basic arithmetic functions that fulfills divider’s needs.

Detail

Line 1~5

I/O Interface.

| Signal Symbol | Signal Name | Signal Description |
| --- | --- | --- |
| src_1 | 32-bits source 1 | Minuend |
| src_2 | 32-bits source 2 | subtrahend |
| result | 33-bits result | Difference, 1 more borrow bit |

Line 6

Wire connection symbolize result = src_1 – src2.

Divisor

Description

Dummy

Detail

Line 1~4

I/O Interface.

| Signal Symbol | Signal Name | Signal Description |
| --- | --- | --- |
| Dvsr_in | Divisor in | In |
| Dvsr_out | Divisor out | out |

Line 5

Wire connection to make out = in.

Control

Description

A counter with 2 in flags, 1 out flag.

Detail

Line 1~6

I/O Interface.

| Signal Symbol | Signal Name | Signal Description |
| --- | --- | --- |
| counting | Counting flag | Input flag represent start |
| rst | Reset flag | Input flag represent reset |
| clk | Clock | Clock |
| ready | Ready flag | Signify the end of operation. |

Line 8~9

Registers

Line 11~18

Use reset register to hold rst signal between 2 posedge clk to avoid racing.

Line 20~31

When running count 32 negedges of clk then set ready flag to notify other component to stop or fetch data.

Remainder

Description

Main module. I use 65 bits representing product in order to handle overflowing result from ALU. The computation is executed on posedge clk to avoid racing condition.

Detail

Line 1~11

I/O Interface.

| Signal Symbol | Signal Name | Signal Description |
| --- | --- | --- |
| clk | Clock | Clock |
| rst | Reset flag | Reset to init state |
| run | Run flag | Run when high |
| Dividend | 32-bits dividend | In when init |
| ALU_result | 33-bits ALU result | With 1 more borrow bit |
| Hi | Hi register | Upper 32 bits |
| Remainder_out | 32-bits Remainder | 34th bit to 65th bit to avoid the right shifting in last iteration |
| Quotient_out | 32-bits Quotient | Output |
| counting | Counting flag | To avoid racing |

Line 12~13

Registers.

| Register Symbol | Register Name | Register Description |
| --- | --- | --- |
| state | State | For FSM |
| Remainder | 65-bits remainder | 1 more bit to handle borrow bit from ALU |

Line 15~18

Wire connection to Remainder_out, Quotient_out, HI, and counting.

Line 21~24

Reset.

Line 26~40

FSM

| State Symbol | State Name | State Description |
| --- | --- | --- |
| 0 | Init | Init Remainer by put Dividend to LO and shift left 1 bit. |
| 1 | run | Try to divide. |

Line 31~39

If ALU_result is negative, shift left 1 bit.

Else, HI = ALU_result, then shift left 1 bit. Finally set Remainder[0] to 1.

Test

Multiplier

tb_CompMultiplier

| Multiplicand | Multiplier | Description |
| --- | --- | --- |
| 000003EB | 00000014 | Common case |
| FFFFFFFF | 00000002 | Multiplicand is extreme value. |
| 00000002 | FFFFFFFF | Multiplier is extreme value. |
| FFFFFFFF | FFFFFFFF | Both are extreme value. |

tb_ALU

Test for functionality of ALU(only addition).

tb_Control

Test for functionality of Control.

(720-60)/20 = 33, 0.5 more for start and 0.5 more for end to avoid racing.

tb_Multiplicand

Test for the functionality of Multiplicand.

tb_Product

Test for functionality.

14h LSB is not 1, shift right.

15h LSB is 1, add to HI, then shift right.

Divider

tb_Control

Test for functionality of Control.

(720-60)/20 = 33, 0.5 more for start and 0.5 more for end to avoid racing.

tb_ALU

Test for functionality

Note that result is 33-bits.

tb_Divisor

Test for functionality.

tb_Remainder

Test for functionality.

The first time ALU_result[32] = 0 -> 28h * 2h = 50h

The first time ALU_result[32] = 1 -> 28h * 2h + 1h = 51h

tb_CompDivider

| Dividend | Divisor | Description |
| --- | --- | --- |
| 00000064 | 00000021 | Common case |
| 00000007 | 00000064 | Common case |
| F0000000 | 0000000F | Dividend is extreme value |
| 0000000F | F0000000 | Divisor is extreme value |
| F0000000 | FFFFFFFF | Both are extreme |
| FFFFFFFF | F0000000 | Both are extreme |

Conclusion and Insights

The most valuable lesson I learnt from this assignment is making report is the most tedious thing in the world. I think whom it may concern will feel the same when reading our naïve reports.

Since Run flag will be set on negedge of clk, it’ll be better if the mul/div operation is on posedge of clk because of the racing problem. Also, because of the counter should be count on negedge to make the last iteration’s output stable (setup time constraint,) this will require 1 more clk to make this happen. Considering the above observation, I make Product/Remainder to tell the control when to start counting. This will 100% make no racing. My structure is not the best in performance, but I think it’ll be relative stabler among all the other students’ structures in this assignment.

During the implementation, I found out that control can just sit there doing nothing but counting, so I did. It’ll need modification if considering merge multiplication and division or even merge all component to a complete ALU, but whatever. The requirement of this assignment doesn’t forbid us to do this.

And there is one more trick I used. I make multiplication/division specific ALU to further simply the control path. As a result, the control just sitting there waiting for set up flags.